PROJECT ENGINEERING LOGBOOK

Robust Dirt-Tolerant Autonomous Line Follower Robot

| Project Name: Dirt-Resistant Line Follower Robot | Document Type: Engineering Development Log |
| --- | --- |
| Current Status: Phase 1 Complete / Phase 2 Design | Date of Output: October 2026 |

1. Project Overview & Objective

The primary goal of this project is to construct a highly reliable autonomous line-following mobile robot capable of navigating tracks obstructed by surface dust, dirt, and light debris. Standard line followers frequently fail or lose track alignment when optical infrared signal reflection is degraded by environmental contaminants. This project employs a multi-sensor fusion approach combining arrayed IR sensing with color detection, alongside a custom mechanical chassis optimized for adverse surface traction and sensor enclosure.

2. Chronological Log Entries

| Log ID & Category | Title & Description | Results & Outcomes | Open Issues / Action |
| --- | --- | --- | --- |
| LOG-001 Hardware / Chassis Assembly | Baseline Prototype Assembly & 4WD Chassis Testing Assembled a standard 4WD smart robot chassis kit featuring dual-deck acrylic plates, four DC gear motors, and standard rubber wheels. Integrated basic dual IR sensors and an Arduino-compatible microcontroller for preliminary mobility and line-following verification. | Successful basic line-following on clean, high-contrast black electrical tape on white background. | Poor maneuverability on tight turns due to 4WD skid-steering friction; erratic behavior on dusty tracks. |
| LOG-002 Sensor Testing & Environmental Validation | Dirt & Contamination Resistance Baseline Testing Evaluated the basic 4WD prototype on a test track covered with varying layers of fine dust, sand, and soil dirt. Analyzed IR reflection analog readings during track contamination. | Identified critical failure modes: simple 2-channel IR sensors lose line position when dirt reduces reflection differential. | IR intensity drops below noise threshold; wheel slip on loose dirt disrupts dead-reckoning trajectory. |
| LOG-003 Sensor Hardware Architecture | Sensor Upgrade Selection: 8-Bit Line Hunter IR Array Upgraded optical sensing architecture from discrete IR modules to an integrated 8-channel (8-bit) Line Hunter IR Sensor Array. The 8-bit array provides high-density spatial resolution, calibrated threshold tuning, and digital/analog output across an array line. | Enhanced spatial tracking allowing PID control algorithm implementation to compensate for missing sensor readings caused by dirt spots. | Requires dedicated high-speed polling and optimized line-position interpolation routines. |
| LOG-004 Sensor Fusion & Calibration | Color Sensor Integration (TCS3200 / TCS34725) Incorporated a specialized RGB color sensor (TCS3200 / TCS34725) positioned alongside the 8-bit IR sensor array. The color sensor provides secondary verification by analyzing surface spectral reflectivity (RGB HSV color space) to differentiate line color from surface dirt tint. | Multi-modal verification: If IR confidence drops due to dirt overlay, the color sensor validates chromatic continuity. | Requires active LED illumination shield to eliminate ambient light interference and dust accumulation on optics. |
| LOG-005 Mechanical Design & Prototyping | Custom Chassis Architecture Design & Specification Initiated design of a custom-fabricated chassis kit to replace the off-the-shelf 4WD kit. Design goals: lower center of gravity, specialized sensor mount with adjustable ground clearance (3-5mm), dust shields for sensor optics, and optimized drive geometry (2WD + rear caster or high-torque track system). | CAD modeling in progress. Custom sensor bracket designed to house both the 8-bit Line Hunter and RGB sensor flush to ground. | Ensuring structural rigidity while maintaining lightweight profile for rapid differential steering. |

3. System Architecture & Upgrades Summary

| Subsystem | Base Prototype (Phase 1) | Upgraded Architecture (Phase 2) |
| --- | --- | --- |
| Chassis Platform | Generic 4WD acrylic smart car chassis kit | Custom-designed high-stability chassis with sensor shroud |
| Primary Line Sensor | Standard 2-channel discrete IR sensor modules | 8-Bit Line Hunter IR Array (High-density multi-point tracking) |
| Secondary Verification | None (Single point IR intensity dependent) | RGB Color Sensor (TCS3200/34725) for chromatic filtering |
| Dirt Handling Strategy | Uncompensated (Fails under poor IR return) | Multi-sensor fusion (Weighted IR + Color verification algorithm) |

4. Next Action Items

▪  Complete CAD modeling and 3D printing / laser cutting of the custom chassis kit.

▪  Implement sensor fusion algorithm (combining weighted readings from the 8-bit Line Hunter IR array and color sensor).

▪  Perform controlled dirt & dust density calibration tests to benchmark line retention accuracy.

▪  Tune PID motor control constants for smooth trajectory correction across contaminated surface patches.